Laboratory work No. 1

«Roots of equations 8.34»

Submitted by

Farrakhetdinova Azaliya

19/09/2025

Problem statement

Beyond the Colebrook equation, other relationships, such as the Fanning friction factor f, are available to estimate friction in pipes. The Fanning friction factor is dependent on a number of parameters related to the size of the pipe and the fluid, which can all be represented by another dimensionless quantity, the Reynolds number Re. A formula that predicts f given Re is the von Karman equation,

1f=4log10(Ref)-0.4

Typical values for the Reynolds number for turbulent flow are 10 000 to 500 000 and for the Fanning friction factor are 0.001 to 0.01. Develop a function that uses bisection to solve for f given a user-supplied value of Re between 2 500 and 1 000 000. Design the function so that it ensures that the absolute error in the result is Ea,d<0.000005.

Mathematical model

Mathematical model of the problem consists of the non-linear algebraic equation of the form

gf=0, 	(1)

where the function f is defined as follows

gf=1f-4log10Ref+0.4

Numerical method

The bisection method will be used for numerical solution of the equation (1). The algorithm of the bisection method can be summarized as the follows:

Step 1. Choose two values, a and b, such that f(a) and f(b) have opposite signs. This means a root lies between a and b.

Step 2. Calculate the midpoint c:

c=a+b2

Step 3. Evaluate f(c): if f(c)=0, then c is the root. If f(c)<ε (ε is prescribed accuracy), then c is the root. If not, continue.

Step 4. Determine which sub-interval contains the root: if f(a) and f(c) have opposite signs, root is in [a,c]; set b=c. Otherwise, set a=c.

Step 5. Repeat steps 2–4 until the interval is small enough (desired accuracy).

Result of